Pulse Code Modulation (PCM)

PCM is a digital communication technique. It takes an analog signal (continuous signal) and converts it into a digital sequence of binary numbers so that the signal can be stored, processed, or transmitted easily.

In simple terms:
PCM = Convert analog signal → digital bits.

PCM is the most widely used digital representation of analog signals in telecommunication, audio, and data transmission systems.

Why PCM is used

- Digital signals are less affected by noise.

- It is easy to store, compress, encrypt, and process digital data.

- Digital systems are more reliable and secure.

- PCM can be transmitted over long distances without distortion.

Main Steps of PCM

PCM works in three main steps:

1. Sampling

Sampling means measuring the value of the analog signal at regular time intervals.

Rule:
fs ≥ 2 × fm
(Nyquist sampling rate)

Where
fs = sampling frequency
fm = highest frequency of the signal

Sampling produces discrete-time values.

Types of sampling:
• Ideal sampling
• Natural sampling
• Flat-top sampling (most common in PCM)

2. Quantization

After sampling, the signal has many amplitude values. These values are still analog. Quantization converts these amplitudes into fixed levels.

Think: Instead of infinite values, choose nearest value from a set of allowed levels.

Types of quantization:

- Uniform quantization (equal step size)

- Non-uniform quantization

  - μ-law

  - A-law

Non-uniform quantization gives better performance for signals with small amplitudes (like speech).

Quantization always introduces a small error called quantization noise.

3. Encoding

The quantized levels are now converted into binary numbers.
Each level is assigned a codeword.

Example:
If there are 8 levels → need 3 bits (2³ = 8).
If there are 16 levels → need 4 bits (2⁴ = 16).

This binary code is the final PCM output.

PCM Block Diagram

- Input analog signal

- Sampler

- Quantizer

- Encoder

- Digital output (bit stream)

Types of PCM

- Natural PCM

- Differential PCM (DPCM)

- Delta Modulation (DM)

- Adaptive Delta Modulation (ADM)

- Adaptive PCM (APCM)

Advantages of PCM

- Very high noise immunity.

- Easy error detection and correction.

- High security using digital encryption.

- Easy signal regeneration at repeaters.

- Compatible with modern digital systems.

Disadvantages of PCM

- High bandwidth requirement.

- More complex than analog communication.

- Quantization noise is unavoidable.

Bandwidth Requirement of PCM

If

- fs = sampling rate

- n = number of bits per sample
Then
PCM bit rate = fs × n

For example:
fs = 8 kHz
n = 8 bits
→ Bit rate = 8,000 × 8 = 64 kbps

Quantization Noise

Quantization introduces an error called quantization noise or quantization error.

For uniform quantization,
Signal-to-Quantization-Noise Ratio (SQNR)
improves by 6 dB for each extra bit.

Non-uniform Quantization (μ-law)

Used mainly in the USA and Japan.
It compresses large amplitude and expands small amplitude.
Gives better quality for speech signals.

PCM Applications

- Telephone networks

- Digital audio (CD, DVD)

- Broadcasting

- Satellite communication

- Data acquisition systems

- Computer sound cards

PCM is the backbone of all digital communication systems where analog signals must be transmitted.

1. Sampling Theorem (Nyquist–Shannon Theorem)

Sampling theorem states:

A continuous-time signal x(t)that contains no frequency component higher than
fm(maximum frequency) can be exactly reconstructed from its samples if the sampling frequency fssatisfies:

fs≥2fm

This required minimum rate is the Nyquist Rate.

A signal limited to fmHz is called band-limited.

Example:
If fm=4 kHz,
then

fs≥8 kHz

2. Sampling Rate

Sampling rate is the number of samples taken per second.

fs=samples per second (Hz)

If x(t)is sampled at interval Ts, then

fs=1Ts

Where
Ts=sampling period.

3. Nyquist Rate

Nyquist rate is the minimum sampling frequency required to avoid aliasing.

Nyquist Rate=2fm

Where
fm=highest frequency of the signal.

If you sample below this rate, overlapping of spectral components occurs, called aliasing.

4. Nyquist Interval

Nyquist interval is the maximum allowed time between two samples.

It is the inverse of Nyquist rate.

TN=12fm

Example:
If fm=2 kHz,

TN=14000=0.25 ms

Meaning:
A sample must be taken at least every 0.25 ms.

5. Sampling Techniques

Sampling is mathematically represented as:

xs(t)=x(t)⋅p(t)

Where
p(t)= sampling pulse train.

Pulse train is periodic with period Ts:

p(t)=n=-∞∞δ(t-nTs)

When multiplied with the analog signal, it creates samples.

Three sampling techniques:

- Ideal Sampling

- Natural Sampling

- Flat-top Sampling

6. Ideal Sampling

Ideal sampling uses delta impulses.

xs(t)=x(t)n=-∞∞δ(t-nTs)

Each sample is an impulse with amplitude equal to the signal value:

xs(t)=n=-∞∞x(nTs) δ(t-nTs)

Characteristics:

- Pulse width = 0

- Pulse height = infinite

- Purely theoretical

7. Natural Sampling

In natural sampling, pulses have finite width (τ), and pulse amplitude follows the signal shape.

Mathematically:

xns(t)=x(t)⋅n=-∞∞Π(t-nTsτ)

Where
Π(t)is a rectangular pulse of width τ.

Characteristics:

- Sample pulse is not flat.

- Top portion of pulse varies with x(t).

- Practical in analog circuits.

8. Flat-top Sampling

Used in PCM systems.

Here a sample-and-hold circuit produces a rectangular pulse whose amplitude is constant during the sample period.

Mathematical representation:

xfts(t)=n=-∞∞x(nTs)⋅Π(t-nTsτ)

In this case, the amplitude:

x(nTs)=constant during the pulse

Characteristics:

- Pulse top is flat.

- Amplitude held for quantizer.

- Used in digital communication to remove amplitude variation during processing.

Aliasing Condition

Aliasing occurs when:

fs<2fm

This causes overlapping of spectra.

A low-pass filter called an anti-aliasing filter is used before sampling.

Quantization

Quantization is the second step of PCM.
It converts the infinite amplitude values of a sampled analog signal into finite, discrete amplitude levels.

In simple words:

An analog sample has unlimited possible values.
Quantization forces each sample to the nearest allowed level.

Thus the output of quantization is discrete in time and discrete in amplitude.

1. Why Quantization Is Needed

After sampling:
We get values like:

3.421 V
1.953 V
−0.872 V
2.100 V
…

These values are continuous.
But computers/digital systems cannot store infinite precision.

Quantization:
• chooses the nearest fixed level
• and assigns that level value to the sample.

This makes digital encoding possible.

2. Quantization Levels

If the quantizer has L levels, then each sample can be assigned only one of these values.

Let

L=2n

where
n=number of bits per sample.

Example:
If n=3, then

L=23=8 levels

Quantization becomes more accurate when Lis large.

3. Quantization Step Size (Δ)

Amplitude range of the input signal is:

Vmax to Vmin

Total range:

R=Vmax-Vmin

If there are Llevels, then the step size is:

Δ=RL

Δ means the spacing between two quantized amplitude levels.

Smaller Δ → better quality
Larger Δ → more distortion

4. Types of Quantization

There are two important types:

A. Uniform Quantization

Δ (step size) is constant for all levels.

Δ=constant

Used for:
• data signals
• low dynamic-range signals

Not suitable for speech because speech has many small amplitude variations.

B. Non-Uniform Quantization

Step size Δ is not constant.
Small amplitudes use small Δ,
Large amplitudes use large Δ.

Two standard companding laws:

- μ-law (North America, Japan)

- A-law (Europe)

This reduces quantization noise for weak signals (like human voice).

Mathematically for μ-law companding:

y=ln⁡(1+μ∣x∣)ln⁡(1+μ)⋅sgn(x)

5. Quantization Error (Quantization Noise)

Quantization always introduces an error called quantization noise.

eq=x-xq

Where
x=actual sample amplitude
xq=quantized level value

Maximum quantization error:

emax=Δ2

The error is unavoidable because we force the value to nearest level.

6. Signal-to-Quantization Noise Ratio (SQNR)

SQNR measures how good the quantization is.

For a uniform quantizer:

SQNR=6.02n+1.76 dB

This shows:
Every extra bit increases quality by 6 dB.

Example:
If n=8,

SQNR=6.02×8+1.76≈49.9 dB

1. Uniform Quantization

Uniform quantization means all quantization steps are of equal size.
Every quantization interval has exactly the same width.

Step Size (Δ)

If total input amplitude range is from

Vmin to Vmax,

and the number of quantization levels is

L=2n,

then the step size is

Δ=Vmax-VminL

Δ is same for every level.

How uniform quantizer works

For each sample x:

- Check which interval the value belongs to.

- Replace the real value with the nearest midpoint of that interval.

Example:
Suppose Δ = 1 V.
If a sample is 2.3 V → nearest level is 2.5 V.

Quantization Error

Error is the difference between actual and quantized values:

eq=x-xq

Maximum error:

∣eq∣max=Δ2

Advantages

• Simple design
• Easy to implement
• Works well for signals with constant amplitude variation

Disadvantages

• Not efficient for speech, because human voice has many small-amplitude signals.
• Low-amplitude parts suffer large relative distortion.

This leads us to μ-law quantization.

2. μ-Law (Mu-Law) Quantization

μ-law is a non-uniform quantization technique.
It uses compression + uniform quantizer, also called companding.

Companding = Compress before quantization → Expand after decoding.

μ-law is used in North America and Japan for telephone PCM (like 8-bit, 8 kHz speech).

Why non-uniform quantization?

Human hearing is more sensitive to small sounds.
Uniform quantization wastes levels on high-amplitude sounds and gives poor resolution for soft sounds.

μ-law makes small signals bigger (compresses dynamic range), so they use more quantization levels.

This reduces error for quiet speech.

Mathematical Formula of μ-Law Compression

Input (normalized between −1 and +1): x

Compression:

y=ln⁡(1+μ∣x∣)ln⁡(1+μ)⋅sgn(x)

Where
μ= compression parameter
Standard value:

μ=255

Explanation of variables:

• x= original sample
• y= compressed sample
• sgn(x)= +1 or −1 depending on the sign
• ln⁡= natural log

What compression does

For small |x|:
ln(1+μ|x|) grows fast → spreads them out → less quantization noise.

For large |x|:
log grows slowly → compresses large signals → saves bits.

μ-Law Expansion (Receiver Side)

After quantization and transmission, the receiver expands:

x=sgn(y)(1μ[(1+μ)∣y∣-1])

Expansion reverses compression, restoring original dynamic shape of speech.

3. Uniform vs μ-Law (Conceptual Difference)

Uniform Quantization

• Δ = constant
• All steps equal
• Good for data signals
• Not ideal for speech
• More noise for low amplitudes

μ-law Quantization

• Δ is small for small amplitudes
• Δ becomes larger for high amplitudes
• Ideal for speech
• Follows human ear sensitivity
• Better Signal-to-Noise Ratio (SNR)

Definition of Line Coding

Line coding is a technique used to convert digital data (usually in binary form) into a digital signal suitable for transmission over a physical communication channel, such as a copper wire, fiber optics, or wireless medium.

- The goal is to represent 1s and 0s (binary data) with voltage levels, pulses, or transitions that can be transmitted reliably.

- It is used in baseband transmission, where signals are sent without modulation.

Mathematically, if the digital data is d[n]where d[n]∈{0,1}, line coding maps this sequence to a signal s(t)that is sent over the channel.

2. Objectives of Line Coding

Line coding serves several purposes:

- Transmit digital data over a channel: Convert binary sequences into electrical signals.

- Maintain synchronization: Ensure that the receiver can determine where each bit starts and ends.

- Control DC component: Avoid a large DC component in the transmitted signal, which is unsuitable for transformers or capacitive coupling.

- Error detection (optional): Some line codes can help detect errors.

3. Basic Properties of Line Codes

- Bandwidth Requirement: The frequency range needed to transmit the signal.

- Power Efficiency: The amount of power required to transmit the signal reliably.

- DC Component: Some line codes produce a DC level; others avoid it.

- Self-synchronization: Ability of the signal to allow the receiver to synchronize with the transmitter.

4. Types of Line Coding

Line coding schemes are broadly classified into three categories:

A. Unipolar Line Codes

- Uses one voltage level (positive or zero) for 1 and zero for 0.

- Simple but has a large DC component.

- Example: Unipolar NRZ (Non-Return to Zero)

| Bit | Signal Level |
| --- | --- |
| 0 | 0 |
| 1 | +V |

Pros: Simple, easy to implement
Cons: Poor synchronization, DC component problem

B. Polar Line Codes

- Uses two voltage levels: one for 1 and the opposite for 0.

- No DC component if signal is balanced.

- Examples: NRZ-L, NRZ-I, RZ

1. NRZ-L (Non-Return to Zero-Level)

- 1 → +V, 0 → -V (or vice versa)

- Level depends on the bit value.

2. NRZ-I (Non-Return to Zero-Inverted)

- Signal inverts when 1 is transmitted; stays same for 0.

3. Return-to-Zero (RZ)

- Signal returns to zero in the middle of each bit period.

- Reduces synchronization problems but requires more bandwidth.

C. Manchester Encoding

- Combines data and clock signal in one waveform.

- Transition occurs at the middle of each bit period:

| Bit | Transition |
| --- | --- |
| 0 | High → Low |
| 1 | Low → High |

- Self-clocking, good for synchronization.

- Used in Ethernet (10BASE-T).

D. Differential Manchester Encoding

- A variation of Manchester encoding.

- Transition at the beginning of bit period indicates 0; no transition indicates 1.

- Robust against polarity reversals.

E. Bipolar Line Codes (AMI - Alternate Mark Inversion)

- 0 → 0V, 1 → +V or -V alternating for each consecutive 1.

- Eliminates DC component and allows easy error detection.

- Example: T1 digital system

5. Key Characteristics and Mathematical Representation

- Bit duration: Tb=1Rb, where Rb= bit rate

- Signal waveform:

s(t)=n=-∞∞d[n]⋅p(t-nTb)

Where p(t)is the pulse shape (rectangular, RZ, etc.)

- Spectral properties: Different line codes have different frequency spectra, affecting bandwidth usage.

6. Comparison of Line Coding Schemes

| Scheme | DC Component | Bandwidth | Synchronization | Example |
| --- | --- | --- | --- | --- |
| Unipolar NRZ | High | Low | Poor | Simple circuits |
| Polar NRZ-L | Medium | Low | Poor | RS232 |
| Polar NRZ-I | Medium | Low | Medium | RS422 |
| RZ | Low | High | Good | Telemetry |
| Manchester | None | High | Excellent | Ethernet |
| Differential Manchester | None | High | Excellent | Token Ring |
| AMI (Bipolar) | None | Medium | Medium | T1 systems |